Single cell barcoded RNA-seq

Edited by Hu Yuqiong & Fan Xiaoying

Revision Date March 2017

Single Cell Barcoded RNA-seq Protocol

Hu Yuqiong & Fan Xiaoying

- Single-cell lysis

Preparation of ERCC spike-in

Making dilution mix (diMix) by adding: 58.2μl H2O + 1.5μl RNase inhibitor + 0.3μl Tween-20

|  | ERCC (3× 108/μl) |
| --- | --- |
| Dilution I | 1μl + 9μl diMix ERCC (3× 107/μl) |
| Dilution II | 1μl + 9μl diMix ERCC (3× 106/μl) |
| Dilution III | 1μl + 9μl diMix ERCC (3× 105/μl) |

The ERCC for a single cell is 3× 104 copies (~2-5% of single cell’s total mRNA).

single-cell lysis buffer

| Component | Volume (μl) (1×) | Total Volume[(n+2)x] |
| --- | --- | --- |
| RNase Inhibitor(40U ul-1) | 0.05 |  |
| 10% Triton X-100 | 0.095 |  |
| Barcode primer(10 uM) | 0.15 |  |
| Nuclease-free water | 1.105 |  |
| dNTP(10 mM) | 0.5 |  |
| ERCC 3x105 | 0.1 |  |
| Total volume | 2 |  |

Pick a single cell to each tube, vortex the tube thoroughly for 30s to 60s, incubate at 72°C for 3 min, then immediately transfer to ice.

- Reverse transcription

| Component | Volume (μl) (1×) | Total Volume[(n+2)x] |
| --- | --- | --- |
| SuperScript II reverse transcriptase(200 U ul-1) | 0.25 |  |
| RNAse inhibitor(40 U ul-1) | 0.125 |  |
| Superscript II first-strand buffer(5x) | 1 |  |
| 0.1M DTT | 0.25 |  |
| Betaine (5M) | 1 |  |
| MgCl2 (1M) | 0.03 |  |
| TSO (100 uM) | 0.05 |  |
| Nuclease-free water | 0.145 |  |
| Total volume | 2.85 |  |

- Add 2.85ul RT mixture to each tube

- Mix the reaction by gently pipetting up and down a few times without forming bubbles.

- Spin down the samples

- 25°C 5min ；42 °C 60min；50°C 30min；70°C 10min, 4°C hold.

- PCR amplify

- Add 7.5ul PCR mix.

| Component | 0.5× Volume (μl) |  |
| --- | --- | --- |
| KAPA HiFi HotStart ReadyMix (2x) | 6.25 |  |
| IS PCR primers (10 uM) | 0.25 |  |
| 3’P2 (10uM) | 0.75 |  |
| Nuclease-free wate | 0.25 |  |
| Total volume | 7.5 |  |

- 95°C 3min, then 4cycles of :98°C 20s,65°C for 30s,72°C for 5min; then 10-16 cycles of :98°C 20s, 67°C 15s, 72°C for 5min; then 72°C 5min, 4°C hold.

扩增的循环数根据细胞类型、大小以及pooling的数量而定。建议扩增完随机测两管的浓度，浓度在10ng/ul-16ng/ul为宜，少于这个范围需要加扩，多于这个范围可以考虑减少循环数。

- Purification

- Pooling the PCR products together. （如果是48管pooling，建议每管取6ul；96管建议3ul）

- Purify with zymo kit once，elute in 50ul H2O.（过柱纯化后抽测一管，参考浓度10ng/ul）

- Purify with twice with 0.8X XP beads, elute in 21ul H2O.（磁珠纯化后一定要测浓度，参考浓度 1.5ng/ul ~ 2.5ng/ul）

- Biotin primer PCR

1）Use 30~40ng DNA as template，use IS primer and Biotin index primer to process PCR.

| Component | 1× Volume (μl) |  |
| --- | --- | --- |
| KAPA HiFi HotStart ReadyMix (2x) | 25 |  |
| IS PCR primers (10 uM) | 2 |  |
| Biotin primer (10uM) | 2 |  |
| DNA template | 21 |  |
| Total volume | 50 |  |

2）95°C 3min, then 4 cycles of : 98°C 20s, 67°C 15s, 72°C for 5min; then 72°C 5min, 4°C hold.

3)  Purify with 0.8X XP beads once，elute in 30ul H2O.（磁珠纯化后抽测浓度，参考浓度10ng/ul~15ng/ul）

- DNA fragment

1）30ul DNA+100ul elution buffer，DNA300 program.for covaris

2）Use zymo kit to purify DNA，elute in 50ul H2O.

3）Purify with 1X XP beads once，elute in 50ul H2O.（磁珠纯化后抽测浓度，参考浓度2ng/ul~4ng/ul）

- DNA biotin enrichment

- Use 10ul C1 beads for each sample.

- Wash the beads once with 200ul 2X B&W buffer，resuspend beads with 50ul 2X B&W buffer.

- 50ul resuspended beads + 50ul DNA fragments，incubate on rotator at room temperature for 45min~1h.

- After incubation, put each tube on magnetic rack, discard the supernatant.

- Add 100ul 1x B&W buffer to wash beads, then add 100ul elution buffer to wash beads again.

- Resuspend beads in 25ul H2O.

KAPA Hyper Prep Kit Illumina® platforms

Library Construction Protocol

1. End Repair and A-Tailing

1.1 Assemble each End Repair & A-Tailing reaction as follows in a tube or well of a PCR plate:

| Component | Volume |
| --- | --- |
| Fragmented, double-stranded DNA | 25 μl |
| End Repair & A-Tailing Buffer† | 3.5 μl |
| End Repair & A-Tailing Enzyme Mix† | 1.5 μl |
| Total volume | 30 μl |

† The buffer and enzyme mix may be pre-mixed and added in a single pipetting step. Premixes are stable for ≤24 hours at room temperature, for ≤1 week at 4°C, and for ≤3 months at -20°C

1.2 Mix thoroughly and centrifuge briefly.（轻微离心，确保磁珠处于悬浮状态）

1.3 Incubate in a thermocycler with the following thermal profile:

| Step | Temp | Time |
| --- | --- | --- |
| End Repair & A-Tailing | 20°C | 30 min |
|  | 65°C | 30 min |
| HOLD | 4°C | ∞ |

1.4 Proceed immediately to the next step.

2. Adapter Ligation

2.1 Dilute adapter as1:10.

2.2 Assemble each Adapter Ligation reaction as follows:

| Component | Volume |
| --- | --- |
| End Repair & A-Tailing reaction product | 30 μl |
| PCR-grade water† | 2.5 μl |
| Ligation Buffer† | 15 μl |
| DNA Ligase† | 5 μl |
| Adapter stock | 2.5 μl |
| Total volume | 55 μl |

† The water, buffer and ligase enzyme may be premixed and added in a single pipetting step. Premixes are stable for ≤24 hours at room temperature, for ≤1 week at 4°C and for ≤3 months at -20°C.

2.3 Mix thoroughly and centrifuge briefly. （轻微离心，确保磁珠处于悬浮状态）

2.4 Incubate at 20°C for 30 min. 4°C pause.

2.5 Add 1.5ul USER enzyme and 37°C for 30min. 4°C pause.

3. Post-ligation Cleanup:

a.将产物置于磁力架上，吸走上清

b.用100ul EB重悬磁珠

c.将重悬后的磁珠置于磁力架上，吸走上清.

d.向磁珠中加入22ul H2O

4. Library Amplification

4.1 Assemble each library amplification reaction as follows:

| Component | Volume |
| --- | --- |
| 2X KAPA HiFi HotStart ReadyMix | 25 μl |
| QP2 primer (10uM) | 1.5 μl |
| short universal primer (10uM) | 1.5 ul |
| Adapter-ligated library | 22 μl |
| Total volume | 50 μl |

4.2 Mix thoroughly and centrifuge briefly. （轻微离心，确保磁珠处于悬浮状态）

4.3 Amplify using the following cycling protocol:

| Step | Temp | Duration | Cycles |
| --- | --- | --- | --- |
| Initial denaturation | 98°C | 45 sec | 1 |
| Denaturation | 98°C | 15 sec | 8-10cycles |
| Annealing† | 60°C | 30 sec |  |
| Extension | 72°C | 30 sec |  |
| Final extension | 72°C | 1 min | 1 |
| HOLD | 4°C | ∞ | 1 |

抽测浓度，参考浓度：12ng/ul

5. Post-amplification Cleanup: Purify twice with 40ul XP beads and elute in 30ul H2O.

注意：从孵育C1开始到KAPA建库扩增结束前，都不可以在-20°C冻存含C1 beads的PCR管。